# SIMATS ENGINEERING

CO4 – AT1 - Article Review- Low

COURSE NAME: Advance Wireless Communication Systems /

5G / 6G Communication Systems

COURSE CODE: ECA5607

COURSE OUTCOME COVERED

CO: 5 - Develop 5G-enabled edge computing and IoT solutions for smart city, healthcare, industrial automation, and intelligent communication applications. (BL6)

Assessment Tool Weightage: 25%

Short description about assessment: Students review recent technical articles to assess critical thinking and awareness of trends.

Dr.

QUESTIONS: 	Total Marks:1        Duration: 90 Minutes

16. Power-Efficient Communication Techniques for IoT Devices in 5G Networks (BL5)

Code of Conduct:

I I . KURUBA ANJI  (192412682) that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

POWER-EFFICIENT COMMUNICATION TECHNIQUES FOR IOT DEVICES IN 5G NETWORKS

# Abstract

The rapid growth of Internet of Things (IoT) devices has created a strong demand for energyefficient communication technologies. IoT devices such as sensors, wearable devices, smart meters, and environmental monitoring systems often operate with limited battery capacity and are expected to function for long periods with minimal maintenance. 5G networks provide high data rates, low latency, massive connectivity, and improved network efficiency, making them suitable for large-scale IoT applications. However, communication activities can consume significant amounts of energy. This article discusses power-efficient communication techniques for IoT devices in 5G networks, including sleep modes, efficient radio resource allocation, low-power wide-area communication, adaptive transmission, energy-aware scheduling, and edge computing. These techniques can reduce energy consumption while maintaining reliable communication and improving the lifetime of IoT devices.

Keywords

IoT, 5G Networks, Energy Efficiency, Power-Efficient Communication, Wireless Sensor Networks, Sleep Mode, Edge Computing, Resource Allocation.

# Introduction

The Internet of Things connects physical devices to communication networks so that they can collect, exchange, and process data. The number of IoT devices is continuously increasing in applications such as smart cities, healthcare, agriculture, industrial automation, and environmental monitoring.

5G networks are designed to support massive numbers of connected devices with high reliability and low latency. However, many IoT devices are battery-powered and have limited computational and communication resources. Frequent data transmission can rapidly drain their batteries.

Therefore, power-efficient communication is an important requirement for 5G-based IoT systems. The main objective is to minimize energy consumption while maintaining the required quality of service, communication reliability, and network performance.

# Need for Power-Efficient Communication

IoT devices may be deployed in locations where replacing or recharging batteries is difficult. Examples include remote environmental sensors, agricultural sensors, industrial monitoring devices, and wearable systems.

Power-efficient communication provides several benefits:

- Increases battery lifetime.

- Reduces maintenance requirements.

- Improves reliability of IoT networks.

- Reduces overall operating cost.

- Supports large-scale deployment of IoT devices.

- Reduces unnecessary energy consumption.

# Power Consumption in IoT Communication

The energy consumed by an IoT device can be broadly divided into sensing, processing, communication, and idle-mode energy.

Among these, wireless communication can be a significant contributor because transmitting and receiving data requires the radio interface to remain active. A simplified relationship for transmission energy can be represented as: Eₜ = Pₜ × Tₜ where:

- Eₜ = transmission energy

- Pₜ = transmission power

- Tₜ = transmission time

Therefore, reducing transmission power and unnecessary transmission time can improve energy efficiency.

# Power-Efficient Communication Techniques

## Sleep Mode and Discontinuous Reception

IoT devices do not always need to remain continuously connected to the network. Sleep modes allow devices to switch off or reduce the activity of their communication circuits when there is no data to transmit or receive.

The device periodically wakes up to check for data or perform communication and then returns to a low-power state.

Advantages:

- Reduces idle power consumption.

- Increases battery lifetime.

- Suitable for sensors that transmit data periodically.

## Adaptive Transmission Power Control

Transmission power can be dynamically adjusted according to the distance between the device and the base station, channel conditions, and required communication quality.

When the channel condition is good, the device can transmit using lower power. Higher power can be used when the channel condition becomes poor.

This technique avoids unnecessary power consumption while maintaining reliable communication.

## Efficient Radio Resource Allocation

5G networks contain multiple users and IoT devices competing for radio resources. Intelligent allocation of spectrum, time slots, subcarriers, and transmit power can reduce energy consumption.

An energy-aware scheduler can assign resources according to:

- Traffic requirements

- Channel quality

- Battery level

- Latency requirements

- Device priority

## Narrowband and Low-Power Communication

Low-power communication technologies are particularly suitable for IoT devices that transmit small amounts of data. 5G supports IoT-oriented communication modes designed for devices with low data-rate and low-power requirements.

These technologies allow IoT devices to communicate efficiently without requiring the high power consumption associated with high-data-rate communication.

## Data Aggregation

Instead of transmitting every sensor measurement separately, multiple readings can be combined before transmission.

For example, several temperature measurements can be processed locally and transmitted as a single summarized value.

This reduces:

- Number of transmissions

- Communication overhead

- Radio operating time

- Energy consumption

## Edge Computing

Edge computing processes data closer to IoT devices instead of sending all raw data to a remote cloud server.

For example, an environmental sensor can process several measurements locally and send only important information to the network.

This reduces communication traffic and can lower the energy required for data transmission.

## Energy-Aware Scheduling

Energy-aware scheduling selects when and how IoT devices should communicate. Devices with low battery levels can be given communication schedules that reduce their energy consumption.

The scheduler can consider both energy consumption and quality-of-service requirements to determine an efficient transmission strategy.

## AI-Based Energy Optimization

Artificial intelligence and machine learning can be used to predict traffic patterns, channel conditions, and device energy requirements.

An intelligent 5G network can use these predictions to:

- Select suitable transmission power.

- Schedule devices efficiently.

- Predict communication requirements.

- Optimize resource allocation.

- Reduce unnecessary transmissions.

# Proposed Power-Efficient 5G-IoT Model

A power-efficient IoT communication system can consist of IoT sensors, an IoT gateway or 5G base station, an edge computing unit, and a cloud server.

The basic operation is:

IoT Sensors → Power-Efficient Transmission → 5G Network → Edge Processing →

Cloud/Application

The IoT sensors collect data and transmit it using optimized transmission power. The 5G network allocates communication resources according to device requirements. Edge computing processes important data locally, while the cloud can be used for long-term storage and advanced analysis.

# Performance Parameters

The efficiency of the proposed system can be evaluated using the following parameters:

Energy Consumption

Measures the amount of energy required to transmit and process IoT data.

Battery Lifetime

Represents the operating duration of a battery-powered IoT device before its battery needs replacement or recharging.

Throughput

Measures the amount of data successfully transmitted through the network per unit time.

Latency

Represents the time required for data to travel from the IoT device to the destination.

Packet Delivery Ratio

It indicates the percentage of transmitted packets successfully received at the destination.

PDR = Successfully Received Packets / Transmitted Packets × 100

Energy Efficiency

Energy efficiency can be represented as the amount of successfully transmitted data per unit of energy:

Energy Efficiency = Successfully Transmitted Data / Energy Consumed

# Applications

Power-efficient 5G-IoT communication can be used in:

- Smart cities

- Smart agriculture

- Healthcare monitoring

- Industrial IoT

- Environmental monitoring

- Smart homes

- Smart grids

- Connected transportation

- Wearable devices

- Remote monitoring systems

# Advantages

- Longer battery life

- Reduced communication energy

- Lower maintenance cost

- Improved network scalability

- Efficient utilization of radio resources

- Reduced network congestion

- Reliable IoT connectivity

- Suitable for large-scale IoT deployment

# Challenges

- Limited battery capacity of IoT devices.

- High energy consumption during wireless transmission.

- Large number of connected IoT devices.

- Dynamic 5G channel conditions and interference.

- Balancing energy efficiency with low latency and high reliability.

- Limited processing and memory resources in IoT devices.

- Security and privacy issues in energy-efficient communication.

# Future Scope

- AI-based energy-efficient resource allocation.

- Energy harvesting using solar, vibration, and RF energy.

- Advanced sleep-mode techniques for longer battery life.

- Intelligent network slicing for IoT applications.

- Edge computing to reduce communication energy.

- Integration of 6G and THz communication for future IoT.

- Machine-learning-based adaptive transmission power control.

# Conclusion

Power-efficient communication is essential for extending the lifetime and reliability of batterypowered IoT devices in 5G networks. Techniques such as sleep modes, adaptive transmission power, efficient resource allocation, data aggregation, edge computing, and AI-based optimization can significantly reduce communication energy consumption. These methods enable sustainable, scalable, and reliable IoT deployment for future smart applications.

# 12.References

- 3GPP, 5G System Architecture and IoT Communication Specifications.

- ITU-R, IMT-2020 5G Network Requirements and Recommendations.

- N. Saeed et al., “Energy-Efficient Communications for IoT Networks,” IEEE Communications Magazine.

- M. Series, “Minimum Requirements Related to Technical Performance for IMT-2020 Radio Interface(s),” ITU-R.

- G. A. R. et al., “Energy Efficiency in 5G Wireless Networks,” IEEE Communications Surveys & Tutorials.

- A. Osseiran et al., 5G Mobile and Wireless Communications Technology, Cambridge University Press.

- M. Agiwal, A. Roy, and N. Saxena, “Next Generation 5G Wireless Networks: A Comprehensive Survey,” IEEE Communications Surveys & Tutorials.

- Ericsson, 5G and IoT Technology Overview.

- Nokia, 5G for Massive IoT and Industrial Applications.

- IEEE, Energy-Efficient Wireless Communication Techniques for IoT Networks.

RUBRICS FOR CALCULATION:

|  |  | Article Review |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Article | 20% | Demonstrates clear and indepth understanding of the article’s purpose, concepts, and arguments | Shows good understanding with minor gaps | Partial understanding; misses key ideas | Lacks understanding of the article |
| Summary | 20% | Provides concise and accurate summary highlighting all key points | Covers most key points with minor omissions | Incomplete or partially accurate summary | Poor or incorrect summary |
| Critical Analysis | 25% | Provides insightful critique with strong reasoning and evaluation | Provides reasonable analysis with some justification | Superficial analysis; limited evaluation | No meaningful analysis |
| Use of Evidence | 15% | Effectively uses examples, data, or references to support review | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Organization | 20% | Wellstructured, clear, and coherent writing with proper language and flow | Generally clear with minor issues | Poor organization; lacks clarity | Disorganized and difficult to understand |